| OUTLINE SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  | 24 February 2026 |  | Client |  |  |  |  |  |  |  |  |  | Project Location |  |  |  | Ghana |  |  |  |  |  |  |  |  |  |  |  |
| Proposal Id |  | Q-1392 |  | Project |  | Parking Structure |  |  |  |  |  |  |  | Building No. |  |  |  | 1 |  |  |  | Option | 1 |  |  |  | Revision |  | 3 |
| Sr.No. | Description |  | PEB SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Building type |  | Mezzanine Building |  |  |  |  |  |  |  |  | Area in m2 |  |  |  | 3740 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Length in m (c/c of steel) |  | 78.00 |  |  |  |  |  |  |  |  | Length wise bay spacing |  |  |  | 2@6.0+10@5.40+2@6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Width in m (c/c of steel) |  | 47.90 |  |  |  |  |  |  |  |  | End wall column spacing |  |  |  | 1@6.0+2@5.980+1@6.0+2@5.980+1@6.0+1@5.980 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Width modules in m |  | NA |  |  |  |  |  |  |  |  | No. of interior columns per frame |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clear height in m |  | NA |  |  |  |  |  |  |  |  | Frames symmetrical |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Roof |  | Slope: NA |  |  |  | Cladding: NA |  |  |  |  |  |  |  | Rainwater drainage: Eave Gutters and Down spouts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Future Expansion |  | NA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Block Wall |  | NA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Wall Cladding |  | NA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Openings |  | Outer side only; Numbers; Type; Size in m (w x h). Doors; NA; NA; NA. Windows; NA; NA; NA. Location of doors & bracing Side Wall Gridlines; 1-2; 2-3; 3-4; 4-5; 5-6; 6-7; 7-8; 8-9; 9-10; 10-11; 11-12; Right End Wall. Front Side Wall; XP; XP; XP. Back Side Wall; XP; XP; XP. End Wall Axes; A-B; B-C; C-D; D-E; E-F; F-G; G-H; H-I; Back Side Wall. Right End Wall. Left End Wall. |
|  | Extensions |  | Eave: NA |  | Gable: NA |  |  | Canopies: Extension in m: NA; Location: NA. Clear height in m: NA; Rainwater drainage: NA. Slope: NA; Soffit panels: NA. | Parapet: NA |  |  |  |  |  |
|  | Accessories |  | Wall Lights: |  |  |  |  |  |  | NA |  |  |  |  |  |  |  |  | Roof Monitor |  |  |  |  |  | NA |  |  | Any other: NA |  |
|  |  |  | Skylights: |  |  |  |  |  |  | NA |  |  |  |  |  |  |  |  | Cage ladder |  |  |  |  |  | NA |  |  |  |  |
|  |  |  | Turbo Ventilators: |  |  |  |  |  |  | NA |  |  |  |  |  |  |  |  | Ridge Ventilators: |  |  |  |  |  | NA |  |  |  |  |
|  | For Frames (Standard codes and manuals used in design - 2010 Metal Building Systems Manual, NAUS07, AISC05, UBC 1997) Design Software: MBS |  | DESIGN LOADS |  |  |  |  |  |  |  | LOCATION - CLIMATIC DATA |  |  |  |  |  |  |  |  | SERVICEBILITY CRITERIA |  |  |  |  |  |  |  |  |  |
|  |  |  | Dead Load in kN/m2 |  |  |  |  |  | 0.10 |  | Max wind speed in m/s |  |  |  |  |  | 36.11 |  |  | Frames |  |  |  |  |  | H/100, L/150 |  |  |  |
|  |  |  | Live Load in kN/m2 |  |  |  |  |  | 0.575 |  | Exposure category |  |  |  |  |  | B |  |  | Purlins |  |  |  |  |  | Span/150 |  |  |  |
|  |  |  | Collateral load in kN/m2 |  |  |  |  |  | 0.05 |  | Max rainfall intensity in mm/hr. |  |  |  |  |  | 150 |  |  | Girts |  |  |  |  |  | Span/120 |  |  |  |
|  |  |  | Any other |  |  |  |  |  | NA |  | Seismic Design category (Coefficient) |  |  |  |  |  | II (0.33) |  |  | Crane Beams |  |  |  |  |  | Span/600 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Max Temperature variation in °C |  |  |  |  |  | 20 |  |  | Mezzanine Beams |  |  |  |  |  | Span/240 |  |  |  |
|  | Mezzanine |  | Location: Ref.Dwg. Loading: Live load in kN/m2: 5 Dead load in kN/m2: 2.5 Collateral load in kN/m2: 0 |  |  |  |  |  |  |  |  |  | Dimensions: Length in m: 78 Width in m: 47.9 Top of the Mezzanine Floor in m: 3.1 Depth of mezzanine slab in mm: 100 Deck sheeting thickness: 1.0 mm |  |  |  |  |  |  |  | Accessories: Staircase Nos.: 2 Location: Ref.Dwg. Type: Double staircase with intermediate landing Handrail: All round |  |  |  |  |  |  |  |  |
|  | Assumptions & Remarks |  | Built-up sections are made from hot rolled plates conforming to ASTM A-572 Gr50 (345MPa) steel. The plates are joined together with double side welding. Sections consist of minimum 5mm for flange & minimum 4mm for web thickness. Cold formed sections thickness 1.5 mm or more are made of hot rolled sheet of Gr50 (345 MPa) steel. Primary structural connections are high strength machine bolts conforming to ASTM A325 or equivalent. Sheeting profile considered is of 1 m standard width, formed from a coil of 1.220 m. Single colour sheeting of up to maximum 9 m lengths is considered. Not considered in scope: Foundation, Block wall (Civil work). Crane Bridge, Trolley, Rail and its installation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |