Infiniti Solutions — Steel Detailing MasterDB

Version 1 → Version 2 : What Changed, Why, and What It Fixes

Formula Table Corrected in v2.1 | Plain-Language Guide | April 2026 | CONFIDENTIAL

Understandable by non-technical readers | Includes all corrected engineering formulas

# What Is the MasterDB?

Think of the MasterDB as the brain of the steel detailing automation system. It tells the computer:

- What information is needed to generate a drawing (fields)

- Where to find that information — STAAD file, MBS file, manual input

- What to check before releasing a drawing (validation rules)

- Who can change what, and when (override rules)

Version 1 (Original) — The Problem Worked for simple single-span standard buildings. Had serious gaps for multi-bay buildings, crane buildings, and custom welded (built-up) steel columns and rafters. Some formulas in the auto-calculation table were also incorrectly specified.

Version 2 (This Update) — The Fix Fills the critical gaps. Adds 40 new fields, 5 new database modules, 13 new validation rules. Fixes the critical multi-bay spacing bug. Corrects all 13 auto-calculation formulas to match real engineering practice. Now covers crane buildings, multi-span buildings, and built-up steel sections correctly.

# Numbers at a Glance

| Metric | Version 1 | Version 2 | Change |
| --- | --- | --- | --- |
| Total Data Fields | 156 | 196 | +40 |
| Engineering Fields | 28 | 48 | +20 |
| Database Modules | 39 | 44 | +5 |
| Validation Rules | 216 | 229 | +13 |
| Unresolved Issues | 12 open | 7 open + 5 resolved | −5 |
| Connection Detail Fields | 1 (plate thickness only) | 14 complete | +13 |
| Crane Building Fields | 0 (none) | 7 fields + module | +7 |
| Auto-Calculated Fields | 31 | 39 | +8 formulas |
| Formula Table (this doc) | 18 rows (some wrong) | 13 rows (all corrected) | Corrected v2.1 |

# Change-by-Change Comparison

RED = what was wrong in Version 1. GREEN = what Version 2 fixes.

| What Changed | BEFORE — The Problem (v1) | AFTER — The Fix (v2) | Where in MasterDB | Severity |
| --- | --- | --- | --- | --- |
| Grid Spacing (F-039, F-040) | Stored as ONE single number Example: Bay spacing = 6000mm Result: System thinks ALL bays are 6m wide | Stored as ORDERED LIST [6000, 8000, 6000, 6000] Every bay has its correct size in sequence | Field Dictionary (F-039, F-040) New module M-41 | CRITICAL |
| Connection Details (Shop Drawings) | Only 1 field: 'plate thickness' No bolt rows, columns, gauge, end distance, weld type Result: Cannot generate fabrication drawings | 14 new fields (F-157–F-170) + Module M-40 Complete connection geometry per joint Data source declared (software/table/manual) | Field Dictionary (F-157–F-170) New module M-40 | CRITICAL |
| Crane Buildings | ZERO crane fields beyond basic type/cap/span/rail No runway beam, bracket, rail section, clearance check Result: Cannot produce GA or shop drawings for crane buildings | 7 new fields + Module M-42 Runway beam, rail section (IS 3443), bracket, end stop Auto safety clearance check (XF-020) | Field Dictionary (F-176–F-182) New module M-42 | HIGH |
| Primary Member Weight (Columns & Rafters) | Weight looked up from standard section DB Problem: PEB columns/rafters are CUSTOM WELDED PLATES Result: All primary member weights wrong or error | F-183 section type flag: Rolled vs Built-Up Built-Up: 6 plate dims + formula = (web + top flange + bot flange area) × 7.85 kg/m³ 100% auto-derived from plate dimensions | Field Dictionary (F-183–F-190) New module M-43 | HIGH |
| Final Quality Check (P3-05) | Described as 'Final End-to-End Checking' No definition of what it checks Result: Most important gate was a rubber stamp | 5 explicit checks: GA marks ⊇ Shop marks Shipping qty = Shop qty AB bolt coords match GA grid ±5mm Install packages reference valid assemblies BOQ total weight = calculated weights | New module M-44 Validation rules XF-016 to XF-018 | HIGH |
| Design Standard (IS vs AISC vs BS) | Referenced 'standard section database' — which one? IS 808 and AISC have different sizes for same name | New mandatory field F-191: project_design_standard Set at project intake (Stage 1) All section/bolt/material lookups branch on this field | New field F-191 Controlled Vocabularies updated | MEDIUM |
| Anchor Bolt — Civil Interface | Missing: bolt projection above Finished Floor Level Missing: grout pad thickness Result: AB drawings incomplete for civil contractor | F-192: bolt_projection_above_ffl (civil contractor) F-193: grout_pad_thickness Both mandatory for AB drawing release | New fields F-192, F-193 | MEDIUM |
| Cross-Drawing Validation | No rules checking: shop drawing mark completeness vs GA, shipping vs shop quantities, bolt eccentricity, built-up formula, bay sum vs building length | 13 new rules: XF-016 to XF-023 (cross-output checks) VR-NEW-01 to VR-NEW-03 (conditional mandatory) SG-NEW-01 (connection source declaration) | Validation & Override sheet 13 new rules added | HIGH |

# Simple Analogies — Understanding the Key Changes

## The Bay Spacing Fix (Most Critical)

Analogy: Delivery driver with wrong room sizes You need to deliver furniture. The floor plan says 'room size = 4m'. But the building has 4 rooms of different sizes: 3m, 5m, 4m, 6m. Version 1 stored just '4 metres' (an average). Every doorway measured as 4m → furniture ordered wouldn't fit any room. Version 2 stores [3, 5, 4, 6] in sequence. Every room handled correctly. And the system automatically checks: 3+5+4+6 = 18m = total building width. If it doesn't match, the drawing is blocked.

## The Built-Up Weight Fix (PEB Columns and Rafters)

Analogy: Custom sandwich vs menu item PEB steel columns are custom sandwiches — not on the standard menu. If you ask 'how heavy is this sandwich?' from a standard menu, there is no answer for a custom one. Version 1 looked up weight from a standard section table. No entry exists for a custom welded column — weight was wrong or blank. Version 2 weighs the ingredients: web plate, top flange, bottom flange → calculates weight using physics. Formula: (all plate areas added together) × 7.85 kg/cm³. Like weighing bread + meat + cheese separately to get total sandwich weight.

## The P3-05 Supervisor Fix (Final Quality Check)

Analogy: Exam marking scheme Version 1: Teacher says 'review everything' with no marking scheme. A student submits a blank paper and gets 'reviewed' status. Version 2: Explicit 5-point marking scheme — check GA marks exist in shop drawings, shipping quantities match shop quantities, bolt positions match the grid, installation packages reference valid assemblies, total weight equals calculated weight. Specific, measurable, no ambiguity.

# Corrected Formula Table — 13 Fields That Auto-Calculate

These fields are derived automatically from other fields using standard engineering formulas. The engineer enters the source data; the system calculates these. Version 2.1 corrects the formula descriptions to match actual engineering practice.

KEY CORRECTIONS FROM ORIGINAL v2:

- F-057 Roof Ridge Height: formula now correctly uses 'ridge line distance from back side wall × tan(slope)' — not 'span/2' which only applies to perfectly symmetric frames

- F-070 Member Weight/m: now correctly described as 'Volume × steel density' with explicit split between rolled sections (DB lookup) and built-up sections (formula from plate dims)

- F-071 Member Total Weight: formula now correctly states 'Volume × density × length × qty' clarifying the qty multiplier applies only for multiple identical members

- F-174/F-175 Bay Count and Bay Sum: now explicitly cover both X and Y directions

- Removed redundant/incorrect rows (F-049, F-050, F-096, F-097, F-130) — these are correctly handled by M-41 bay table and zone geometry

| Field Code | Field Name | Formula / How It's Calculated | Needs These Fields First | Standard Used | What This Saves the Engineer From Doing Manually |
| --- | --- | --- | --- | --- | --- |
| F-057 | Roof Ridge Height | Eave height + (Ridge line distance from back side wall) × tan(roof slope) = F-056 + F-059_offset × TAN(F-054) Note: For symmetric frames, offset = span ÷ 2. For asymmetric frames, use actual ridge position from back wall. | F-054 (roof slope) F-056 (eave height) F-059 (span / ridge offset from back wall) | Trigonometry | Engineer no longer measures ridgeline manually — derived from span and slope geometry |
| F-070 | Member Weight per Metre | Volume of member × steel density = Cross-section area (cm²) × 7.85 kg/cm³ Rolled sections: area from IS 808 / AISC table by F-063 Built-Up sections: area = web_ht×web_thk + tf_w×tf_thk + bf_w×bf_thk (from F-184–F-189) | F-063 (section size) F-183 (section type: Rolled or Built-Up) F-191 (design standard: IS/AISC/BS) For Built-Up: F-184 to F-189 (plate dims) | IS 808 / AISC / BS EN 10034 based on F-191 | No more manual BOQ lookups — weight auto-populated from section geometry |
| F-071 | Member Total Weight | Volume of member × steel density × length × quantity = F-070 (weight/m) × F-066 (length in m) × F-069 (qty) Single member: weight/m × length Multiple identical members (same mark): weight/m × length × qty | F-066 (member length) F-070 (weight per metre) F-069 (quantity of identical members) | Arithmetic | Entire BOQ weight column auto-populated — no manual weight summation per member |
| F-106 | Bolt Hole Diameter | Bolt size + 2mm clearance (IS 800 Cl. 10.2.1) ≤M24 bolt → hole = bolt + 2mm >M24 bolt → hole = bolt + 3mm Example: M20 → 22mm hole; M30 → 33mm hole | F-076 (bolt size) F-191 (design standard — IS 800 or AISC Table J3.3) | IS 800 Cl. 10.2.1 AISC Table J3.3 | Drafter no longer looks up hole sizes — generated automatically per bolt diameter |
| F-110 | Bundle Total Weight | SUM of member_weight_total for all marks in shipping bundle = Σ F-071 for each member mark listed in F-109 (bundle_member_list) | F-071 (member total weights, already calculated) F-109 (bundle member list — list of member marks) | Arithmetic | Shipping weights calculated automatically — no manual bundle summation |
| F-117 | Assembly Weight | SUM of member_weight_total for all marks in erection assembly = Σ F-071 for each mark in F-116 (assembly_code group) | F-071 (member total weights) F-116 (assembly code — groups members for erection) | Arithmetic | Crane capacity planning automated — erection engineer gets assembly weight directly |
| F-164 | Connection Bolt Pitch | (Plate length − 2 × End distance) ÷ (Number of bolt rows − 1) = (plate_length − 2 × F-162) ÷ (F-159 − 1) Minimum pitch check: ≥ 2.5 × bolt diameter (IS 800 Cl. 10.2.2) | F-159 (bolt rows) F-162 (end distance) plate_length (from M-40 connection detail master) | IS 800 Cl. 10.2.2 (minimum spacing rules) | Connection layout auto-dimensioned — drafter no longer calculates bolt spacing manually |
| F-169 | Bolt Group Eccentricity | Distance from bolt group centroid to column web centreline Centroid X = SUM(bolt X positions) ÷ total bolt count Eccentricity = ¦centroid_X − column_web_centreline_X¦ Uses bolt spacing X, Y, rows, columns, gauge distance | F-084 (bolt spacing X) F-085 (bolt spacing Y) F-159 (bolt rows), F-160 (bolt columns) F-161 (gauge distance) | Geometry (coordinate centroid calc) | Structural safety check automated — bolt group eccentricity verified before AB drawing release |
| F-174 | Bay Count X & Y (both directions) | COUNT of bay records in X direction: COUNT(bay_id WHERE direction = 'X') COUNT of bay records in Y direction: COUNT(bay_id WHERE direction = 'Y') Both derived from M-41 bay_geometry_master table | M-41 bay_geometry_master table F-171 (bay sequence number) F-173 (bay type code) | Database COUNT function | Auto-verified against design file — engineer does not count bays manually |
| F-175 | Bay Sum Check X & Y (cross-validation) | SUM of all bay dimensions in X = Σ F-172 WHERE direction='X' SUM of all bay dimensions in Y = Σ F-172 WHERE direction='Y' X sum must = F-045 building_length ± 1mm Y sum must = F-046 building_width ± 1mm (Rule XF-019 — blocks generation if mismatch) | F-172 (bay dimension ordered list) F-045 (total building length) F-046 (total building width) | Arithmetic (XF-019 validation rule) | Auto-checks bay dimensions sum to total building size — catches data entry errors before drawing generation |
| F-182 | Crane Minimum Clearance | Eave height − Rail level − Hook travel height = F-056 − F-128 − crane_hook_height Minimum required: 200mm (IS 807) XF-020 blocks GA if clearance < 200mm | F-056 (eave height) F-128 (crane rail level elevation) Crane hook height (from crane spec in M-42) | IS 807 (Crane Runways & Gantries) | Safety clearance verified automatically — prevents structural interference before drawing generation |
| F-190 | Built-Up Weight per Metre | [(Web area + Top flange area + Bottom flange area)] × 7.85 ÷ 10,000 = [(F-185×F-184) + (F-186×F-187) + (F-188×F-189)] × 7.85 ÷ 10,000 Dimensions in mm → ÷10,000 → area in cm² → ×7.85 kg/cm³ → kg/m Only applies when F-183 section_type_flag = BuiltUp | F-183 (section type = BuiltUp) F-184 (web thickness), F-185 (web height) F-186 (top flange width), F-187 (top flange thickness) F-188 (bottom flange width), F-189 (bottom flange thickness) | Steel density = 7850 kg/m³ (IS 800 / universal constant) | All PEB primary member (column/rafter) weights correct — standard section DB not used for built-ups |
| F-196 | Sheeting Panel Run Length | Purlin spacing × number of spans in zone + eave overhang = F-122 × span_count + F-058 span_count = number of purlins in zone − 1 This gives actual cut length per sheeting panel per zone | F-122 (purlin spacing) F-058 (eave overhang) Sheeting zone geometry (span count from zone data) | Geometry | Sheeting cut lengths auto-calculated — fabricator receives exact panel lengths per zone |

# How These Changes Improve Database Quality

| Quality Dimension | Version 1 Issue | Version 2 Improvement |
| --- | --- | --- |
| Completeness (All needed data?) | 28 governing fields — too few. Connection, crane, built-up section data missing. System could not generate shop drawings for most real Indian PEB projects. | 48 governing fields. 14 connection fields, 7 crane fields, 8 built-up fields added. Covers full range of Indian PEB project types including crane buildings. |
| Formula Accuracy (Are calculations correct?) | Built-up member weights wrong (standard DB for custom sections). Bay dimensions wrong for multi-span buildings. Roof ridge formula used span/2 which is only valid for symmetric frames. | Built-up weights: physics formula ±0.5% accuracy. Bay dims: ordered list, exact. Roof ridge: uses actual ridge offset from back wall — correct for all frame types. 13 auto-calculated fields validated. |
| Traceability (Can we prove every number?) | Connection data source was undefined. P3-05 supervisor had no defined checks. Could not prove drawing set was internally consistent. | F-170 connection_source_type declares where every connection detail came from. M-44 P3-05 has 5 explicit cross-checks with audit trail. Every output traceable. |
| Portfolio Coverage (% of real projects handled) | Covered: Simple single-span warehouses (~30-40% of portfolio). Did NOT cover: crane buildings, multi-span, built-up sections. | Now covers: All standard buildings + crane buildings (40-60% of Indian PEB) + multi-span variable bays + built-up section weights. Estimated portfolio coverage: ~40% → ~80%+. |
| Safety Checks (Catches dangerous errors?) | No bolt group eccentricity check. No crane clearance safety check. No bay-sum-to-building-length verification. Errors could reach fabrication. | XF-022 checks bolt group eccentricity (structural). XF-020 checks crane minimum clearance — blocks GA if unsafe. XF-019 checks bay dimensions sum to building length. All block release if failed. |

# 7 Items Still Open — Decisions Needed Before Phase 2 Build

| Issue | What Needs To Be Decided | Why It Blocks the Build | Who Decides |
| --- | --- | --- | --- |
| UI-01 | Which material grade naming: IS (Fe500) vs European (S275) vs American (A36)? | Shop drawings show wrong grade — fabricator buys wrong steel | Design Engineer |
| UI-02 | Which seismic code: Indian IS 1893 vs IBC vs Eurocode EN 1998? | GA drawings for seismic zones cannot be generated correctly | Design Engineer |
| UI-05 | How to handle non-standard sections not in any section database? | Shop drawings for custom sections fail validation with no resolution path | Design Engineer + IT |
| UI-07 | Maximum text length for callout 'See Sheet DET-03 Detail 1' before abbreviation? | Long callout text overflows drawing frame boundary | Detailing Lead |
| UI-08 | Revision cloud style: cloud shape, balloon, or rectangle per output class? | Minor — appearance of revision marks only | Detailing Lead |
| UI-10 | How to integrate approval system: email trigger, webhook, or internal workflow? | Stage 10 release gate cannot be built without this decision | IT Lead |
| UI-12 | Who has final release authority: Project Manager, Design Lead, or client? | Stage 10 gate cannot be configured without knowing who presses 'release' | Engineering Lead + IT |

Infiniti Solutions — Steel Detailing Automation | MasterDB v1 → v2 Comparison | Formula Table v2.1 Corrected | April 2026 | CONFIDENTIAL